EQUITY-AWARE POWER LOAD-SHEDDING

(A linear-programming approach to fair electricity allocation during blackouts)

SUMMARY

When electricity supply falls short of demand, the way power is rationed matters as much as how much power exists. Most real-world load-shedding still rotates power on a fixed schedule that ignores what each zone actually needs: a hospital and an empty office building are treated the same. This project builds and tests an alternative: a linear-programming (LP) optimizer that allocates scarce grid capacity to minimize total harm (a single score summing, across every zone and hour, how much unmet need remains after allocation, weighted by each zone's criticality, so failing a hospital counts far more than failing an empty office), while enforcing a hard fairness guarantee that no zone is left without power for more than a set number of consecutive hours.

Across a simulated 20-zone, 48-hour blackout, the optimized schedule reduced total harm by 66.8% relative to a fixed-rotation baseline, eliminated all hospital power shortfalls, and brought two previously fully dark cold-storage and water-pumping zones to full coverage: all while maintaining zero fairness violations. A stress test further shows the honest limits of this approach: as available capacity drops toward 35%, the model's advantage narrows, and even hospitals begin to see real shortfall hours.

# THE PROBLEM

## Importance

Rolling blackouts and load-shedding are a recurring reality in many parts of the world, particularly regions with under-capacity grids or unstable generation. During these events, the harm is not evenly distributed: a few hours without power is an inconvenience for a residential household, but the same outage can spoil vaccines and food in cold storage, halt water pumping stations that entire communities depend on, or force a hospital onto diesel backup it may not have enough fuel to sustain. The stakes scale directly with how essential a zone's function is: which is precisely the dimension that need-blind rationing ignores.

## Tractability

Load allocation under a hard capacity constraint is a linear-programming (LP) problem: minimize a harm objective subject to capacity, demand, and fairness constraints. LP solvers are mature, fast, and well understood, which means a working, provably optimal solution is achievable within a short build window.

## Neglectedness

Despite the tractability, most real-world load-shedding schedules are still fixed rotations, determined by feeder/circuit grouping rather than by real-time need. This is largely an operational-simplicity default, not a technical limitation. Utilities rotate blocks of the grid because it's simple to communicate and administer, not because it's optimal. A relatively small amount of additional modelling (criticality scoring, an LP layer, a fairness constraint) can meaningfully close that gap without requiring new hardware.

THE MATHEMATICS

The core of the system is a linear program solved once per scheduling horizon (all 48 hours at once, not hour-by-hour), because the fairness constraint spans multiple hours and an hour-at-a-time solver could not see it.

## Effective need

Every zone's priority is derived from a single formula that combines how important the zone is with how much it actually still needs from the grid:

effective_need(z, h) = criticality_score(z) × max(0, demand(z, h) − backup_capacity(z))

This ensures a zone with strong on-site backup (e.g. a hospital generator that fully covers its demand) is not treated as needing grid power at all, while a zone with identical criticality but no backup is prioritized correctly.

## Decision variable and objective

- Decision variable: x[z,h] ∈ [0,1], the fraction of zone z's hourly demand served by the grid at hour h. It is continuous rather than binary, reflecting that real allocation is a matter of degree (partial supply, brownout-style), not a strict on/off switch.

- Objective: minimize total harm, defined as Σ effective_need(z,h) × (1 − x[z,h]) across all zones and hours, i.e., minimize the criticality-weighted, backup-adjusted unmet need.

## Constraints

- Capacity: at every hour, total power allocated across all zones cannot exceed the available grid capacity for that hour.

- Demand ceiling: no zone can be allocated more than 100% of its own hourly demand.

- Fairness (hard constraint): no zone may go more than N consecutive hours with less than a minimum served fraction. N ("the fairness cap") is a parameter tested at 4h, 6h, and 8h.

The fairness rule is deliberately implemented as a hard constraint rather than a soft penalty. A hard constraint gives an absolute, provable guarantee, but can render the problem infeasible if capacity is too low relative to the cap. We accept that trade-off because a guarantee that can silently erode under pressure defeats the purpose of calling the system "equity-aware."

SOLUTION

The system was built and validated end-to-end as five sequential stages, each producing an artifact consumed by the next:

- 1. Dataset generation: 20 synthetic zones across five categories (hospitals, cold storage/water, schools, commercial, residential) over a 48-hour horizon, each with a category-shaped demand curve and randomized backup capacity.

- 2. Needs module: the effective_need and outage-tracking formulas, covered by unit tests against hand-calculated edge cases before being trusted downstream.

- 3. Optimizer: the LP described above, solved with PuLP, returning a provably optimal allocation schedule.

- 4. Baseline: a fixed-rotation scheduler with no awareness of criticality or backup, used as the honest point of comparison, operating under the identical capacity constraint.

- 5. Evaluation: both schedules scored with an identical harm formula, plus a sensitivity sweep across shrinking capacity (55% / 45% / 35%) and varying fairness caps (4h / 6h / 8h).

A companion Streamlit dashboard makes these results explorable: capacity- and fairness-cap sliders rerun the optimiser live, updating headline stats (harm reduced, worst outage streak, hospital shortfall), a per-zone outage comparison chart, and an hour-by-hour zone status view. The dashboard is deployed and viewable at:
https://k9cdd5afnggekbzhpvmiof.streamlit.app/

RESULTS

At the project's baseline scenario (55% of peak demand available, 4-hour fairness cap):

| Metric | Baseline (fixed rotation) | Optimized |
| --- | --- | --- |
| Total harm (55% capacity) | 18,113.6 | 6,006.3 |
| Harm reduction | -- | 66.8% |
| Hospital shortfall hours (of 96) | 12 | 0 |
| Cold storage/water outage (of 48h) | 48 | 0 |
| Fairness violations | 0 | 0 |

Table 1. Headline comparison between the naive fixed-rotation baseline and the LP-optimized schedule.

Two results stand out as the strongest evidence that the model responds to genuine need rather than simply favouring a category label. First, cold storage and water-pumping zones (which carry no backup power) went from being completely unpowered for the entire 48-hour simulation under the baseline to fully served under the optimized schedule. Second, the dataset intentionally includes two hospitals with different backup strength: one with a generator that fully covers its own demand (correctly left mostly untouched by the grid allocation) and one with weak backup that was underserved 12 hours under baseline and fully protected under the optimized schedule. The model treats the two hospitals differently because their actual need differs.

Sensitivity analysis
To test the model honestly rather than only in its best-case configuration, we re-solved the LP across nine combinations of grid capacity and fairness cap:

| Capacity | Fairness cap | Total Harm | Hospital Shortage (of 96h) |
| --- | --- | --- | --- |
| 55% | 4h | 6,006.3 | 0 |
| 55% | 8h | 5,371.5 | 0 |
| 45% | 4h | 10,470.3 | 3 |
| 45% | 8h | 9,517.1 | 0 |
| 35% | 4h | 16,850.4 | 27 |
| 35% | 8h | 15,417.0 | 17 |

Table 2. Total harm and hospital shortfall hours across capacity and fairness-cap settings.

Two trends emerge. Loosening the fairness cap consistently reduces total harm, because the optimizer gains freedom to concentrate power where need is highest for longer stretches. Still, it does so by allowing a longer legitimate dark stretch for some zone, which is the direct cost of that efficiency gain. More importantly, as capacity shrinks from 55% toward 35%, the optimizer's relative advantage narrows, and hospitals begin to see real shortfall hours (rising to 27 of 96 possible hospital-hours at 35% capacity, 4-hour cap). This is a limit because below a certain point, there simply is not enough electricity for even the smartest allocation to protect every critical zone.

LIMITATIONS

- The dataset is entirely simulated. A real deployment would require sourcing zone-level demand and backup data from smart meters, utility self-reporting, or a tiered manual classification process.

- Criticality scores are assigned by category (hospital = 10, residential = 1, etc.), a simplification of the more nuanced, context-dependent judgments a real utility or emergency-management body would need to make, ideally in consultation with affected communities.

- The current model solves a 20-zone, 48-hour problem in under a second; a city-scale deployment with thousands of zones and a rolling real-time horizon would need to be tested for solver performance at that scale before any live re-optimization claim could be made.

- "Total harm reduced" is partly circular, since it is the LP's own minimized objective. We therefore also report two independent, non-circular metrics which are consecutive outage hours and hospital shortfall hours, so the headline number is not the only evidence of improvement.

- The fairness constraint can render the LP infeasible if the cap is too strict relative to available capacity; a production system would need a defined fallback behaviour for that case, which this project has not yet built.

# THEORY OF CHANGE

The path from this project to real-world impact runs through utilities, disaster-response agencies, and micro-grid operators who currently rely on fixed-rotation load-shedding by default rather than by informed choice. The core claim (a criticality- and backup-aware LP with a hard fairness guarantee outperforms naive rotation on both harm reduction and protection of essential services) is directly testable against any utility's own outage and infrastructure data, without requiring new hardware. The realistic next step is not deployment but validation: partnering with a utility, disaster-response NGO, or municipal grid operator willing to share (or help synthesize more realistically) real zone-level demand and backup data, to test whether the harm-reduction and fairness gains shown here hold at real scale.

TOOLS USED

1. Python: pandas/NumPy for data handling and PuLP (CBC solver) for the optimization. 
The dashboard itself is built in Streamlit, written in PyCharm, and deployed via Streamlit Community Cloud.

AI tool-- CLAUDE AI for:

- Code scaffolding: the dataset generator, needs module, optimizer, baseline, and evaluation scripts

- Exploration and experimentation: the fairness cap/capacity sensitivity testing

- Dashboard planning: the mockups used before building the real Streamlit app

CONCLUSION AND FUTURE WORK

This project demonstrates that a fairness-constrained linear program can meaningfully outperform naive fixed-rotation load-shedding: cutting simulated harm by 66.8% and eliminating hospital shortfalls, while remaining honest about where that advantage breaks down under more severe scarcity. Future work would prioritize: real-world data partnerships to replace the synthetic dataset; scale-testing the solver against realistic city-sized zone counts; and exploring a soft-penalty fallback for the fairness constraint so a real deployment degrades gracefully rather than failing outright when the hard constraint becomes infeasible.